# Name & Student ID

Name: Abdul Yassir Sarajzada

Student ID: 20553901

# Title

Project planning document

# Outline of the Problem to be Solved

The aim of this project is to code a drawing robot to write a certain piece of text onto a flat surface. The robot receives the commands in the form of G code, where it understands how to write each character by being told the X and Y coordinates of each character and when to place the pen on the page. The shapes of the letters come from a separate file which contains the specific code for every character.

The user will be asked to input the font size, from which the robot will read the piece of text it is required to write from a separate file. The robot will then process each character separately, following the commands for the X and Y coordinates as well as the Z coordinate to tell it when to place the pen on the surface. The G code is sent over a serial connection on an Arduino which then controls the robot arm.

The robot is required to be able to move to the correct starting position when a new character is being written, to correctly lift and lower the pen when it is needed and reset back into the starting coordinates when the text has been written.

# Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| textInput | string | This variable stores the text it is tasks with writing |
| scaleFactor | float | Converts the units 18 to the desired height (4-10mm) |
| textHeight | double | The user inputted height (4-10mm) |
| positionX | double | Tracks the horizontal offset to position new characters accurately |
| positionY | double | Tracks the vertical offset to position new characters accurately |
| fontData | struct | Stores the drawing instructions for each character |
| penPosition | int | Tracks if the pen is on the page or not |

# Function Declarations

- Float calculatedScaleFactor(float userInput)

Parameters:

- userInput – the inputted height

- return value – scaleFactor

- void loadFontData(struct fontData)

parameters:

- fontData – the array that stores the instructions on how to write the character

- return value – none

- void transferGcode(char* Gcode)

parameters:

- Gcode – The G code string

- Return value – none

- Void writeText(char* textInput, float scaleFactor)

Parameters:

- textInput – The inputted text from the user

- scaleFactor – scale in which the characters will be written in

- return value – none

- char writeCharacter(char A, float scaleFactor, float* positionX, float* positionY)

Parameters:

- A –  character to write

- scaleFactor – scale in which the characters will be written in

- positionX – pen position in X coordinates

- positionY – Pen position in Y coordinates

- return value – A

- void main()

- parameters – none

- return value - none

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| calculatedScaleFactor | Minimum height | 4 | 4/18 = 0.22 |
| calculatedScaleFactor | Maximum height | 10 | 10/18 = 0.56 |
| writeCharacter | Write single character | A | Character written correctly |
| writeText | Write the desired text | Hello world | Words written correctly |
| main | End condition | Runs program | Program ends with pen up in (0,0) position |

# Flowchart(s)

# AI Statement

Used ChatGPT - Prompt used “Can you explain what this assignment is asking me to do in plain English (attached the CompEngSoftwareAssignmentPlanning word document)